TeleMaster

630C Product family (4-lines)

External Programming Guide

Full Option [Version: 6.04]

Caution:

***Please make sure you have read and understand all the detail in this programming guide before adjusting any option or begin the Programming process.***

***Every time when this Programming Software turned ON, all the items will be in default option selection or value, please make sure you have do all the necessary adjustment correctly before programming the pager.***

***If by any chances that the adjustment was messed up. You just have to turn OFF this programming Software and restart all over again.***

Getting Started:

630C-Series Pager Programming Software and Interface Package, combined with an IBM personal computer (or compatible) provides the convenience and choice of combinations to program 630C-Series pagers to meet individual specific requirements.

To avoid misunderstanding or to obtain the best results, please take a few minutes to read this Programming Guide carefully and follow step by step to operate.

Equipment Required:

- IBM Personal Computer or Compatible PC:

With ３８６ CPU or higher, DOS version 5.0 or higher, a parallel port, 1.4MB floppy disk drive and 1024KB of available RAM.

- 630C-Series Programming Interface Fixture. (Provided by TeleMaster Paging Division)

- Interface Cable.

25 pins (RS232) cable connects the computer's parallel port and the 630C-Series Programming Interface Fixture.

- Pager Programming Software.

Software is provided on a 1.4MB diskette, with Programming Software Serial Number.

(Provided by TeleMaster Paging Division)

- Power Source.

A new 1.5V AA size Alkaline Battery.

Equipment Setup:

- Turn OFF the computer power.

- Refer to (Figure 1) to plug the appropriate connector of the Interface Cable into the 630C-series Programming Interface Fixture, and connect the other end to the computer's parallel port.

- Install a new 1.5V AA size Alkaline Battery into the Fixture.

Software Setup:

- Insert the Pager Programming Software Diskette into the computer's floppy drive.

- Turn ON the computer power.

The computer will automatically start up with the 630C-Series Programming Software.

***In order to start up this Programming Software, you must turn ON the computer     with the Programming Software diskette inside the floppy drive. Therefore your     computer's ＢＩＯＳ must be start up by floppy drive in first priority. If you have any      problem, please refer to your computer's mother broad instruction manual.***

Preparing the Pager for Programming or Reading:

- Turn OFF the pager by following the instruction in the User Manual.

- Open the battery cover and take off the battery (if there is a battery inside). Make sure to take off the rubber strip at the position of Figure 5. (If this is not the first time of programming to the Pager).

- Please read the Programming Software Serial Number on the Pager and make sure it is the same as the Numbers display on the upper right corner of the Programming      Software Start Up Screen before you move on to the next step. If the Programming       Software Serial Number is not the same, please check with your local dealer.

- Please follow the (Figure 2 & 3) to insert and hold the Pager in position inside the      Fixture. When the Pager is in the correct position, it will start the self-testing function.

- Before starting to program the Pager, please make sure it had finished Self-Testing function and turned into standby mode.

- Basic Software Operation Method:

- Follow each function Description Control Key Hot Key to pull up each Function Adjustment Window.

- Use the ↑, ↓, ← or → arrow keys to move the cursor up, down, left or right to the item that you wanted to adjust.

- Use the Insert & Delete keys to switch option.

- Use  0  ~  9  number keys to directly key in selection or 7 digits Cap Code.

- Use F2 Save or F3 Save +1 to program the pager with the new option adjustment.

- Use F4 Read to read parameters inside the pager.

- Use F10 Quit to turn OFF this Programming Software.

- Map1 Cap Code 1~6 parameter setup (Ctrl F1):

- Reception Enable (ENB) or Disable (DSB) of each Cap Code.

- 7 digits of each Cap Code.

- Paging Source Tone (Sub-address) A, B, C & D of each Cap Code:

  - Mode = Message will be storage in Personal or Public Data Bank through each Paging Source Tone.

  - Alert = Pager will alert according to the selected alert option (User Set Alert mode, Standard POCSAG Tone, Short Beep or Mute) when received message through each Paging Source Tone.

  - Type = Pager will decode message according to the selected Message Decoding Format (4-Numeric or 5-Alphanumeric) when received message through each Paging Source Tone.

**Please only select 4-Numeric or 5-Alphanumeric for 4-Lines alphanumeric pager.

- Map2 Cap Code 1~6 parameter setup (Ctrl F2):

***Please do not change any default option or selection.***

- Map3 Cap Code 1~6 parameter setup (Ctrl F3):

***Please do not change any default option or selection.***

- Read message display mode (Ctrl F6):

***Please do not change any default option or selection.*** Default selection should be 3.

- Icon display setup parameter (Ctrl F7):

- Reception Icon trigger timing. “   “ Extensional display timing after received each paging signal. Please only adjust Map1 timing.

(Option: No Delay, Delay 30 second, Delay 60 second or Delay 180 second)

- Display Public data bank ICON? Display selection. Please keep it by Yes.

- Paging Source Tone display timing.

(Option: 0.25, 0.5, 0.75, 1.0, 1.25, 1.5, 1.75 or 2 second)

- Power ON display Title (Ctrl F8):

- Power ON display type.

(Option: User set, 1=Voice Message, 2=Rainy Day, 3=Happy Holidays or

4= Happy Birthday)

- "User set" display Title [MILLENNIUM]: Maximum 15 characters can be display.

Please key in the new title by keyboard.

- Operation Setup (Alt F1):

***Please do not change any default option or selection.***

- Select standard operation mode? = Yes.

- Select stock pager operation mode? = No.

- Key-tone ON? = Yes.

- Return to standby screen after the last key operation? = Yes.

- [Reading message] LCM display OFF after the last key operation? = Yes.

- Standby screen & LCM display delay timing after the last key operation.

= 18 second.

- Back light ON delay timing after the last key operation? = 10 second.

- RF Receiver setup (Alt F2):

- Baud Rate selection. (Option: 512bps, 1200bps & 2400bps)

- Frequency for synthesized receiver.

Please key in the new frequency according to and within the frequency range of the synthesized receiver.

Please only adjust Map-1 frequency.

- Data Input: Please make sure you do under this combination format before you select option (Normal or Reverse).

  - 1XX.XXXX MHz frequency: Select Reverse for Positive paging reception.

  - 1XX.XXXX MHz frequency: Select Normal for Negative paging reception.

  - 4XX.XXXX MHz frequency: Select Normal for Positive paging reception.

  - 4XX.XXXX MHz frequency: Select Reverse for Negative paging reception.

- RF Enable Preactive Width.

Please "Do Not" tries to adjust this figure.

- PLL Enable Preactive Width.

Please "Do Not" tries to adjust this figure.

** (4) & (5) item will shelf adjust according to the Baud Rate selection.

- 4-Lines display setup (Alt F3):

***Please do not change any default option or selection.*** Default selection should be 3.

- No. = 3 digits.

- <BEL> = Enable.

- <FF> = Enable.

- <CR> = Enable.

- <LF> = Check LF.

- File option selection (Alt F):

- New Code plug.

Press Enter key to reset all the option selection to default setup.

Press Esc key to escape.

- Load Power ON display text file.

Press Enter key to load ".txt" file for Power ON title.

Press Esc key to escape.

- Save Power ON display text file.

Press Enter key to save ".txt" file for Power ON title.

Press Esc key to escape.

- Programming the pager (F2 Save or F3 Save +1):

- Before pressing the F2 Save or F3 Save +1, please press Ctrl F1 to go back to the windows of Map1 address setting.

- By pressing the F2 Save key to program the pager, all the parameter will stay the same.

- By pressing the F3 Save+1 key to program the pager, all the parameter will stay the same except the 7 digits Cap Code which high lighted by the cursor will automatically add 1 digit to its value.

- Then use the ↑, ↓, ← or → arrow keys to move the cursor up, down, left or right to the correct Cap Code position which you wanted to increase by 1 digit after you had pressed F3 Save +1.

- Press F2 Save or F3 Save +1 to program the new adjusted parameter to the pager.

- After function key F2 Save or F3 Save +1 was pressed, there will be two result display:

  - If the programming to the Pager is successful, the upper right corner will display Write OK! Wording.

  - If in any case the programming to the Pager has occur ERROR the computer screen will display the window of Try Again & Cancel and that mean you have to try again or cancel this programming operation. And if you had tried more than three times without successfully programming the Pager, please read and follow the Trouble Shooting Guide instructions before contacting your dealer.

- After finished programming the Pager, please follow the Figure 4. direction to pull out the Pager from the Fixture.

***You must follow the direction or otherwise you may bend the Testing Pins and the Fixture can no longer function.***

***Remember to put back the rubber strip in the position of Figure 5. in order to protect the programming port from oxidation or others contamination.***

- Programming Confirmation F4 Read:

Press F4 Read to read the programmed parameter of the pager.

- Turn OFF the programming Software F10 Quit:

Press F10 Quit to turn OFF the Programming Software.

***After you had finished using the Fixture and the Diskette, please put them into a clean & dry plastic bag in order to protect the programming port from oxidation or others contamination until the next time.***

- Trouble Shooting Guide:

- Programming Software does not start up:

Check if the computer's ＢＩＯＳ being set to start up by floppy drive in first priority and if the diskette had correct insert into the floppy drive.

- Pager didn't power ON while inserted into the Fixture:

  - If the Cable correctly connected to the computer parallel port and the                       Fixture.

  - If there is a new 1.5V AA size Alkaline Battery installed at the back of the                       Fixture or is the battery voltage below 1.3V (Please replace with a new battery).

  - Has the rubber strip been taken off?

  - If the programming port of the Pager or the Testing Pins on the Fixture been oxidized or bended.

  - If the Pager can be power up by install a 1.5V AAA size Alkaline Battery. (If the Pager doesn't turn ON send it back for servicing)

<<If everything's works fine, please contact your dealer. >>

- The Transmission of EEPROM data failed window display:

  - If the Programming Software Serial Number on the Pager match with the                     Programming Software.

  - If there is a new 1.5V AA size Alkaline Battery installed at the back of the Fixture or is the battery voltage below 1.3V (Please replace with a new battery).

  - If the programming port of the Pager or the testing pins on the Fixture been oxidized or bended.

  - If the computer meet the Equipment Required Requirements.

  - If the computer's ＢＩＯＳ being set to output data from LPT1.

<<If everything's works fine, please contact your dealer. >>

- Pager does not function after programming has been performed:

  - If the Address Code has not been changed.

  - If the Address Code has not been Enable.

  - If the Pager has not been successfully programmed. By following the same                      process of programming but press F4 Read key to read the parameter that inside                      the Pager instead of F2 Save or F3 Save+1 key. If the parameter is not correct please adjust the incorrect parameter and reprogram the Pager again.

- If anything else's beside the above-mentioned items happen, please do not try to disassemble the Pager or the Fixture and please contact your dealer.

- Maintenance Information:

Maintenance of the 630C-Series Programming Package consists of:

    - Keeping the Fixture and the diskette clean, and making sure that to put them into a plastic bag in order to protect the pager's programming port and the testing pins from oxidation or others contamination.

    - Do not store the Package until extreme tempers, humidity or direct sunlight.

    - Do not apply strong shock to the Fixture or drop the Fixture.

    - Avoid bending the Programming Software Diskette.

    - The Testing Pins can be cleaned with a soft, dry cloth but be very careful not to bend the Testing Pins. Do not use any solvents to clean the Testing Pins.

    - If necessary, a damp (not wet) cloth can be used to clean the outside of the Fixture.

    - Please don't try to disassemble, repair or modify the Fixture yourself.