## Programming Manual for Topic POCSAG Pager

# Programming Kit Installation

- Connect the programmer port to the computer printer port.

- Connect the 6-pin connectors to the 6-pin halls in battery chamber of pager.

- Press the kit properly connected to the pin halls.

# Programming

- Cap Code Mode –

### To edit cap code

- In DOS mode, execute “topic66.exe” file.

- Input passwords “CapsLock” and press [Enter] button.

- On the next screen, the data file name is displayed. Press [Enter].

- To edit programming details, press [E] button. By pressing [↑] or [↓], you can select cap code, message type and call type.

- If you want to change cap code, move the cursor to the cap code and type the desired cap code. And press [Enter].

- Our pager supports dual frame cap code. So, 6 cap codes should be in dual frame.

- When you change message type and call type, press [Space] button. Whenever you press [Space], the type is changed.

- Message type : “U” means personal message and “M” means maildrop message.

- Call type : “A” means alpha message and “N” means numeric message.

- After you set the programming details, press [Esc] button.

### To write cap code

- Connect the 6-pin connector of programmer to 6-pin halls inside pager properly and press the kit.

- Press [W] button to write cap code into pager. If the writing is successful, “Write OK” is displayed.

When you write the cap code continuously, press [A] or [Enter].

### To read cap code

- Press [R] button to read cap code.

- The programmed cap code of pager will be displayed on the screen.

### To move into option mode

If you want to change option parameter, press [Tab] button.

### To close programming

Press [Q] to exit.

- Option Mode –

### To edit option

- To edit programming option, press [E] button. By pressing [↑] or [↓], you can select bit rate, frequency, frame select, auto space, sequential lock, melody no., maker logo, start logo, end logo, antenna display, signal polarity and out of range.

- Bit rate, frame select, auto space, sequential lock, antenna display, signal polarity and out of range will be changed by [Space] button.

- Bit rate : The pager is produced at 1200bit rate in factory. When you change bit rate to 512 or 2400, the components should be changed also.

- Frequency : There is no meaning in frequency change. Do not change.

- Frame select : Please select the cap code frame matching to the cap code you programmed on cap code mode.

- Auto space : If you want to set auto spacing (auto syllable), select “On”.

- Sequential lock : You can select “Off” for sequential lock disable or select certain minutes for enable.

- Melody no. : Do not change this option.

- Maker logo : You can set 14 characters for power on/off logo.

- Start logo : Do not change this option.

- End logo : Do not change this option.

- Antenna display : you can set the antenna display type on the screen what you want.

- Signal polarity : you can select “Normal” or “Inverse” for polarity.

- Out of range : You can set out-of range time from 0 to 8 minutes.

- After you set all the programming option, press [Esc].

### To move into cap code mode

Press [Tab] to return to cap code mode.

# C. To close programming

Press [Q] to exit.